Initial Command Functions

1.	  Announce arrival

2.	  Take command

3.	  Size-Up

A.	Type of Incident ______________________________________

- B.	Name of Ship _________________________________________

- C.	Location of Ship ______________________________________

- D.		Access Route ________________________________________

- E.		Condition of Piers _____________________________________

- F.	Construction _________________________________________

- G.	Occupancy (Type/Service of Ship) ____________________________

- H.	Apparatus/Manpower __________________________________

- I.		Life Hazard __________________________________________

- J.		Water Supply ________________________________________

- K.	Auxiliary Appliances (Ship Systems) _________________________

- L.	Street Conditions (Access) _______________________________

- M.	Weather _____________________________________________

- N.	Exposures ___________________________________________

- O.	Area ________________________________________________

- P.	Location of fire _______________________________________

- Q.	Time of Day __________________________________________

- R.	Height/HAZMAT ______________________________________

- S.	Observable Conditions ________________________________

_______________________________________________________

- T.	Resources Immediately Available ________________________

_______________________________________________________

- U.	Pre-planned Resources ________________________________

- V.	Tide, Wind & Current __________________________________

- W.	 ____________________________________________________

4.		Make situation report

5.		Establish Command Post

6.		Request additional alarms if needed

7.		Establish a staging area

8.		Notify US Coast Guard/MIRT

Phone #: __________________________

Request Marine Firefighting Coordinator

Request Maritime Incident Response Team

9.		Establish safety zones

Ashore: Local Law Enforcement/Facility Security

Waterway: USCG/Marine Police

10.		Request SCBA compressor unit or cascade

11.		Notify Local Port Authority

12.		Determine your strategic objectives

A.	Rescue

- Fire Attack

  - Exposures

  - Containment

  - Extinguishment

  - Dewatering

- Property & Environmental

  - Booming

  - Air Monitoring

  - HAZMAT

Initial Response Organization

## Incident Commander

1st Alarm Units

## Staging

2nd  Alarm Units

13.		 Make division assignments from first alarm units and give strategic assignments.

14.		Assign a Logistics Officer to provide supplies to operational divisions.

Transitional incident organization

## Incident Commander

CG On Scene Coordinator

Liaison

### Marine Fire

Coordinator

Staging

#### Operations

##### Logistics

MIRT

#### Fire Containment

#### Rescue

#### Water Supply/ICS

Priority 1 – Rescue

1.		Contact ship's officers and terminal manager.

2.		Determine number of casualties.

A.	Number of people involved:	__________________

B.	Location of all people involved:__________________

C.	Dead/Injured:	__________________

D.	Trapped:	__________________

E.	Not accounted for:	__________________

3.		Determine your rescue objectives.

- Evacuate civilians

- Search for victims

- Extricate victims

- Shelter-in-place

- Triage

Initial Unified Command Organization

Unified Command

###### Liaison

Safety

Planning

Operations

Logistics

CG Marine Fire Coordinator

MIRT

Water Supply

HAZMAT

Rescue Branch

### Staging

Fire Attack Branch

###### Rescue

###### Deck Division

###### Pier Division

###### Triage

###### Water Division

Treatment

RIT Division

#### Transportation

Continue developing your staff as needed.

1.		Assign Operations Officer to direct and coordinate all tactical operations.

2.		Assign Safety Officer to identify hazardous and unsafe conditions to insure protection of personnel.

3.	 	Assign Information Officer to release information to the press.

4.		Assign Planning Officer to forecast probable course of events and project alternative courses of action.

5.		Assign Liaison Officer as a contact for responding agency representatives.

Think of these assignments as jobs and not as people.  If you can't find someone to do it, you will have to do them yourself.

Priority 2 - Fire Attack

1.		Contact ship's officers

A.	Captain

B.	Chief Mate

C.	Chief Engineer

D.	Other Officers

E.	Ship's Owner or local agent

F.	Interpreter

2.		Determine status of the fire.

A.	Request ship's fire plan.

B.	Request cargo storage plan and general arrangement plan.

C.	Request dangerous cargo manifest

D.	Location of fire

E.	Type of area involved.

F.	Type and quantity of material burning.

G.	Primary cargo.

H.	Location, type and quantity of hazardous material aboard.

I.	When and how did fire start?

J.	What has crew done to fight fire?

K.	Fire protection systems onboard.

L.	Areas protected by systems.

M.	Have systems been used to fight fire?

N.	Access to fire area.

O.	Access to exposed spaces.

P.	International shore connection.

Q.	Status of ventilation system.

R.	Location and status of fire resistant bulkheads, fire zones, and w/t doors.

S.	Status of ship's pumps and fire main system.

T.	Flooding or instability.

U.	Projected progression of the fire.

V.	______________________________________________________________

W.	______________________________________________________________

X.	______________________________________________________________

3.		Communicate updated status report.

4.		Strike greater alarms or activate mutual aid plans if needed.

5.		Fire attack objectives.

A.	Protect external exposures.

Warehouses

Piers

Waterfront facilities

B.	Remove endangered shipping.

C.	Cool the exterior house and hull.

D.	Secure ventilation air and fire dampers.

E.	Effect natural and/or mechanical ventilation.

F.	Secure w/t doors & hatches.

G.	Set fire boundaries.

H.	Activate ship's fire systems.

I.	Augment ship's fire main system through international shore connection.

J.	Position hose lines at the fire boundaries.

K.	Apply from involved area.

L.	Coordinate interior attack.

M.	Off load cargo.

N.	Break burning fuel on surface of water into patches and drive away from ships and piers with fireboat monitors.

O.	Flood, scuttle, ground, or beach.

6.		If you intend to put water on the ship, request coast guard strike team or other agency with dewatering equipment at the earliest possible time.  Don't wait until the ship begins to list.

7.		Request resources to support fire attack objectives.

- Greater alarms

- MIRT

- Mutual aid

- Fire boats

- Rescue boats

- Coast Guard vessels

- Helicopters

- Tug boats

- Cranes

- Foam nozzles & eductors

- Bulk foam

- Bulk CO₂

- Divers

- Oxy-Acetylene cutting torches

- Welders/cutters

- Lighting equipment

- Longshoremen

- Cats & Loaders

- Switch engine

- Shipboard firefighting specialist

- Fuel

- Food

- Marine inspectors, US Coast Guard

Unified Command Organization

UC (USCG/MASTER/FIRE)

Legal Officer

Safety Officer

Information Officer

Liaison Officer

##### Logistics

Planning

##### Operations

Finance

STAGING

Fire Ops

Salvage Ops

Rescue Ops

HAZMAT OPS

Security Ops

Integrate Local MIRT into appropriate positions as required.

Priority 3 - Property and Environmental Conservation

1.		Determine property conservation objectives.

- Pollution Control.

  - Booming

  - Skimmers

- HAZMAT

  - Air Monitoring

  - Cargo

  - Ship’s Stores

- Salvage.

  - Dewatering

2.		Request resources to support conservation objectives.

- Commercial salvage master.

- Oil Spill Removal Organizations

- Qualified Individual (Tank Vessels)

- State Environmental Agencies

- Oil skimmers.

- Oil containment booms.

- US Coast Guard Strike Team.

- Marine environmental protection specialists.